## SPECIFICATION SHEET

| PRODUCT | # CARMINEX™ HIGH TINTING STRENGTH 52% |
| --- | --- |
| DESCRIPTION | It is a Carmine Lake presented under the form of a Red dye powder, obtained from the aqueous extract of cochineal (Dactylopius coccus costa), wich under certain conditions gives a complex aluminium calcium salt, known under the name NATURAL CARMINE. This dye has a strong covering power that gives special high red tone in final applications, and is composed of very small-sized particles. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ High Tinting Strength 52% is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SYNONYMS | Natural red 4 CI 75470. Chemical Abstracts Service (CAS) registry number 1390-65-4. EEC CODE E 120. |
| CONCENTRATION | 52% as Carminic Acid. The concentration is adjusted with natural carriers GMO free. |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX II (FCCII) |
| SOLUBILITY | Carminex™ High Tinting Strength 52% is not soluble in neutral water, diluted acids, ethanol, and propylene glycol but completely soluble in alkaline water. |
| STABILITY | Highly resistant to light oxidation, change in pH and microbiological attacks. Very stable toward heat up to 130 centigrade degrees, but unstable above that temperature. |
| APPLICATION | Specially elaborated for using in DRY MIXES in the food industry (dairy desserts, processed cheese, ice cream, jams, sauces, processed meat, confectionery, snacks, alcoholic beverage, baked goods), cosmetics and pharmaceutical industries. The FDA lists Carmine among colours permitted in foods, cosmetics and drugs, without limitation and exempt from certification. |
| CONSERVATION | Two years at normal temperature. Store in a cold and dry location (5-15°C). No refrigeration needed. Prevent from humidity. |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 300 Moisture : less than 15% Mercury : less than 1 ppm Lead : less than 5 ppm Arsenic : less than 0.1 ppm Other heavy metals : less than 20 ppm. |
| RESIDUAL SOLVENTS | NONE |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms Less than 3/g. Salmonella Not detected Vibrio-cholera Not detected E coli Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100 Kg. inside a fibre drum. |
| AVAILABILITY | 4000 Kg/month. |

## SPECIFICATION SHEET

| # PRODUCT | # CARMINEX™ CUSTOMIZED # SHADE 52% |
| --- | --- |
| # DESCRIPTION | # It is a Carmine Lake presented under the form of a Red dye powder, obtained from the aqueous extract of cochineal (Dactylopius coccus costa), which under certain conditions gives a complex aluminium calcium salt, known as NATURAL CARMINE. This high covering strength dye is composed of very small-sized particles. The shade, available in reddish, yellowish and blueish, is elaborated with respect to the special requirements of the clients. |
| # CERTIFICATION | # Sephardi Kosher Organization |
| # APPROVAL | # Carminex™ Customized shade 52% is in full compliance with The directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| # SYNONYMS | Natural red 4 CI 75470. Chemical Abstracts # Service (CAS) registry number 1390-65-4. EEC CODE E 120. |
| # CONCENTRATION | 52% as Carminic Acid. The concentration is # adjusted with natural carriers GMO free. |
| # ANALYSIS | All analysis approved under the FOOD # CHEMICAL CODEX II (FCCII) |
| # SOLUBILITY | # Carminex™ Customized shade 52% is not soluble in neutral water, diluted acids, ethanol, and propylene glycol but completely soluble in alkaline water. |
| # STABILITY | Highly resistant to light oxidation, change in pH # and microbiological attacks. Very stable toward heat up to 130 Centigrade degrees, but unstable above that temperature. |
| # APPLICATION | # Specially elaborated for using in DRY MIXES in the food industry (dairy desserts, processed cheese, ice cream, jams, sauces, processed meat, confectionery, snacks, alcoholic beverage, baked goods), cosmetics and pharmaceutical industries. The FDA lists Carmine among colours permitted in foods, cosmetics and drugs, without limitation and exempt from certification. |
| # CONSERVATION | # Two years at normal temperature. Store in a cold and dry location (5-15°C). No refrigeration needed. Prevent from humidity. |
| PHYSICO-CHEMICAL # SPECIFICATIONS | Particle Size : 100% mesh 300 Moisture : less than 15% Mercury : less than 1 ppm Lead : less than 5 ppm Arsenic : less than 0.1 ppm # Other heavy metals : less than 20 ppm. |
| # RESIDUAL SOLVENTS | # NONE |
| MICROBIOLOGICAL # STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected # E coli : Not detected |
| # PACKING | Double polyethylene bags of 5, 25, 50 or 100 # Kg. inside a fibre drum. |
| # AVAILABILITY | # 4000 Kg/month. |

## SPECIFICATION SHEET

| PRODUCT | CARMINEX™ HIGH CONCENTRATION 65% |
| --- | --- |
| DESCRIPTION | It is a Carmine Lake presented under the form of a Red dye powder, obtained from the aqueous extract of cochineal(Dactylopius coccus costa), wich under certain conditions gives a complex aluminium calcium salt, known under the name NATURAL CARMINE. The colouring principle is the Carminic Acid, and is available in high concentrations, even more than 65%. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ High Concentration 65% is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SYNONYMS | Natural red 4 CI 75470. Chemical Abstracts Service (CAS) registry number 1390-65-4. EEC CODE E 120. |
| CONCENTRATION | 65% as Carminic Acid. The concentration is adjusted with natural carriers GMO free. |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX II (FCCII) |
| SOLUBILITY | Carminex™ High Concentration 65% is not soluble in neutral water, diluted acids, ethanol, and propylene glycol but completely soluble in alkaline water. |
| STABILITY | Highly resistant to light oxidation, change in pH and microbiological attacks. Very stable toward heat up to 130 Centigrade degrees, but unstable above that temperature. |
| APPLICATION | Food (dairy desserts, processed cheese, ice cream, jams, sauces, processed meat, confectionery, snacks, alcoholic beverage, baked goods), cosmetics and pharmaceutical industries. In general, it is suitable for any application for which Carmine High Concentration is needed. The FDA lists Carmine among colours permitted in foods, cosmetics and drugs, without limitation and exempt from certification. |
| CONSERVATION | Two years at normal temperature. Store in a cold and dry location (5-15°C). No refrigeration needed. Prevent from humidity. |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 200 Moisture : less than 10% Mercury : less than 1 ppm Lead : less than 5 ppm Arsenic : less than 0.1 ppm Other heavy metals : less than 20 ppm. |
| RESIDUAL SOLVENTS | NONE |
| MICROBIOLOGICA STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100 Kg. inside a fibre drum. |
| AVAILABILITY | 4000 Kg/month. |

### SPECIFICATION SHEET

| PRODUCT | CARMINEX™ WATER SOLUBLE 50% |
| --- | --- |
| DESCRIPTION | It is a Hydrosoluble Carmine available under the form of a dark spray-dried dye powder, obtained from the alkaline aqueous extract of cochineal (Dactylopius coccus costa), which under certain conditions gives a complex aluminium calcium, salt, known under the name NATURAL CARMINE. The colouring principle is the Carminic Acid. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ Water Soluble 50% is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SYNONYMS | Natural red 4 CI 75470. Chemical Abstract Service (CAS) registry number 1390-65-4. EEC CODE E 120 |
| CONCENTRATION | 50% as Carminic Acid. The concentration is adjusted with natural carriers GMO free. |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX II (FCCII). |
| SOLUBILITY | Carminex™ Water Soluble 50% is completely soluble in water. |
| STABILITY | Highly resistant to light oxidation, change pH and microbiological attacks. Very stable toward heat up to 130 Centigrade degrees, but unstable above that temperature. |
| APPLICATION | Food Industry : dairy desserts, processed cheese, ice cream, jams, sauces, processed meat, confectionery, snacks, alcoholic beverage, baked goods. The FDA list the carmine among colours permitted in foods, without limitation and exempt from certification. |
| CONSERVATION | Two years at normal temperature. Stores in a cold and dry location (5°-15°C). No refrigeration needed. Prevent from humidity. |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 200 Moisture : Less than 10% Mercury : Less than 1 ppm. Lead : Less than 5 ppm. Arsenic : Less than 0.1 ppm. Other heavy metals : Less than 20 ppm. |
| RESIDUAL SOLVENTS | NONE |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100 Kg. inside a fibre drum. |
| AVAILABILITY | 4000 Kg/month. |

# SPECIFICATION SHEET

| PRODUCT | CARMINEX™ LIQUID 5%. |
| --- | --- |
| DESCRIPTION | Obtained from a water-alcoholic solution of Carmine, alkalinised with ammonia to prevent microbiological attacks. |
| CERTIFICATION | Sephardi Kosher Organization |
| SYNONYMIS | Natural red 4 CI 75470. Chemical Abstract Service (CAS) registry number 1390-65-4 EEC CODE E 120. |
| APPROVAL | Carminex™ Liquid 5% is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| CONCENTRATION | 5% as Carminic Acid. |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX II (FCCII) |
| MISCIBILITY | Carminex™ Liquid 5% is completely miscible in water with no solid residuals. |
| STABILITY | Highly resistant to light oxidation, change pH and microbiological attacks. Very stable toward heat up to 130 Centigrade degrees, but unstable above that temperature. |
| APPLICATION | Food industry: dairy desserts, processed cheese, ice cream, jams, sauces, processed meat, confectionery, snacks, alcoholic beverage, baked goods. The FDA list the carmine among the colours permitted in food without limitation and exempt from certification. |
| CONSERVATION | Two years at normal temperature. Stores in a cold and dry location (5° - 15°C). No refrigeration needed. Prevent from humidity. |
| PHYSICO-CHEMICAL SPECIFICATIONS | pH : Not less than 11 Mercury : Less than 1 ppm. Lead : Less than 5 ppm. Arsenic : Less than 0.1 ppm. Other heavy metals : Less than 20 ppm. |
| RESIDUAL SOLVENTS | NONE. |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g. Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Sealed plastic bottles of 5, 20, 50 and 200 Kg. |
| AVAILABILITY | 4000 Kg/month. |

# SPECIFICATION SHEET

| PRODUCT | CARMINEX™ 90% PROTEIN FREE |
| --- | --- |
| DESCRIPTION | Red brilliant powder obtained from aqueous extract of cochineal (Dactylopius Coccus Costa), after eliminating proteins, resins, fats ans waxes. This extract, under certain conditions, gives a complex aluminium – calcium salt known under the name Natural Carmine. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ 90% Protein Free is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SYNONYMS | Natural Red 4. C.I. 75470. Chemical Abstracts Service (CAS) registry number 1260-17-9. EEC CODE E 120. |
| CONCENTRATION | Not less than 90% as Carminic Acid. |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX II (FCCII) |
| SOLUBILITY | Carminex™ 96% Protein Free is not soluble in neutral water but completely soluble in alkaline water. |
| STABILITY | Highly resistant to light oxidation, change in pH, microbiological attacks. Very stable toward heat up to 130 centigrade degrees, but unstable above that temperature. |
| PROTEINS | Less than 1% |
| ADVANTAGES | Carminex™ 90% Protein Free is completely free of other unknown cochineal extract solubles present in low purity carmines that may cause not desired reactions in some food color apllication. Its purity characteristic allows formulator to know exactly what it is added. Besides, this product has an extremely easy solubility in alkaline water, with strong color value, perfect shade and high penetrating power that make Carminex™ 90% Protein Free leading color for food application. |
| APPLICATION | Food (soups, meat products, spices), pharmaceutical industries. The FDA lists the carmine among colours permitted in foods, without limitation and exempt from certification. |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX III (FCC III). |
| METALS | Mercury : less than 1 ppm Lead : less than 5 ppm Arsenic : less than 0.1 ppm Other heavy metals : less than 20 ppm |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g. Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PARTICLE SIZE | 100% through mesh 200 |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100 Kg. inside a fiber drum. |
| AVAILABILITY | 1000 Kg/month. |

### SPECIFICATION SHEET

| PRODUCT | CARMINEX LIQUID 3.7% PROTEIN FREE |
| --- | --- |
| DESCRIPTION | Dark red alcaline aqueous solution obtained from aqueous extract of cochineal (Dactylopius Coccus Costa) after eliminating proteins, resins, fats and waxes. This extract, under certain conditions, gives a complex aluminum – calcium salt known under the name Natural Carmine. |
| CERTIFICATION | Sephardi Kosher Organization |
| SYNONYMS | Natural red 4 CI 75470. Chemical Abstracts Service (CAS) registry number 1390-65-4. EEC CODE E 120. |
| CONCENTRATION | Not less than 3.7% as Carminic Acid. |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX II (FCCII) |
| SOLUBILITY | Carmine is completely soluble in water, alcohol and propylenglygol |
| STABILITY | Highly resistant to light oxidation, change in pH and microbiological attacks. Very stable toward heat up to 130 centigrades degrees, but unstable above that temperature. |
| PROTEINS | Less than 0,2% |
| ADVANTAGES | Liquid carmine solution 3.7% protein free is completely free of other unknown cochineal extract solubles present in low purity carmines that may cause not desired reactions in some food color application. Its purity characteristic allows formulator to know exactly what it is added. Besides, Liquid carmine solution 3.7% protein free has an extremely easy solubility in water with strong color value, perfect shade and high penetrating power that make it leading color for food application |
| APPLICATION | Food (soups, milk, yoghurt, meat products, spices), pharmaceutical industry. The FDA list the carmine as colourant permitted in foods, cosmetics and drugs without limitation and exempt from certification. |
| METALS | Mercury : less than 1 ppm. Lead : less than 5 ppm. Arsenic : less than 0.1 ppm. Other heavy metals : less than 20 ppm. |
| MICROBIOLOGICAL ANALYSIS | Total plate count : Less than 1000/g Mould count : Less than 100/g Yeast count : Less than 100/g Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli. : Not detected |
| PACKING | Sealed plastic bottles of 5, 20, 50 or 200Kg. |
| AVAILABILITY | 5000 Kg./month. |

### SPECIFICATION SHEET

| PRODUCT | #### CARMINEX™ OIL SOLUBLE 10% |
| --- | --- |
| DESCRIPTION | It is an Oil Soluble Carmine available under the form of a dark red liquid solution, obtained from extract of cochineal (Dactylopius coccus costa), which under certain conditions gives a complex aluminium calcium, salt, known under the name NATURAL CARMINE, emulsified with polyglicerine ester. The colouring principle is the Carminic Acid. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ Oil Soluble 10% is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SYNONYMS | Natural red 4 CI 75470. Chemical Abstract Service (CAS) registry number 1390-65-4. EEC CODE E 120 |
| CONCENTRATION | 10% as Carminic Acid. The concentration is adjusted with natural carriers GMO free. |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX II (FCCII). |
| SOLUBILITY | Carminex™ Oil Soluble 10% is completely miscible in oil with no solid residuals. |
| STABILITY | Highly resistant to light oxidation, change pH and microbiological attacks. Very stable toward heat up to 130 Centigrade degrees, but unstable above that temperature. |
| APPLICATION | Food Industry : fats, oils, margarine, snack foods, salad dressings, condiments. The FDA list the carmine among colours permitted in foods, without limitation and exempt from certification. |
| CONSERVATION | Two years at normal temperature. Store in a cold and dry location (5°-15°C). No refrigeration needed. Prevent from humidity. |
| METALS | Mercury : Less than 1 ppm. Lead : Less than 5 ppm. Arsenic : Less than 0.1 ppm. Other heavy metals : Less than 20 ppm. |
| COMPOSITION | Carmine : 10% Polyglycerine ester : 36% Soy oil : 54% |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Sealed plastic bottles of 5, 20, 50 or 200 Kg. |
| AVAILABILITY | 4000 Kg/month. |

# SPECIFICATION SHEET

| PRODUCT | CARMINEX™ ACID 90% |
| --- | --- |
| DESCRIPTION | It is the free colouring matter Carminic Acid with a purity not less than 90%, obtained from an aqueous-alcoholic extract of cochineal (Dactylopius Coccus Costa). It is free of metals like calcium and aluminum. This is the difference with carmine. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ Acid 90% is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SYNONYMS | Natural Red 4. C.I. 75470. Chemical Abstracts Service (CAS) registry number 1260-17-9. EEC CODE E 120. |
| USE | It is used as colourant in foods, drugs, cosmetics and also as a raw material for acid stable red colour production |
| SOLUBILITY | Carminex™ Acid 90% is 100% soluble in water in all range of pH, in alcohol and propylene glycol. |
| STABILITY | Highly resistant to light oxidation, change in pH, microbiological attacks. Very stable toward heat up to 130 centigrade degrees, but unstable above that temperature. |
| APPLICATION | Food Industry: jams, sauces, confectionery, alcoholic beverage, soft drinks. The FDA lists the carmine among colours permitted in foods, without limitation and exempt from certification. |
| CONSERVATION | Two years at normal temperature. Store in a cold and dry location (5-15ºC). No refrigeration needed. Prevent from humidity. |
| COMPOSITION | Carminic Acid 90.00% Moisture 8.00% Other natural solubles 2.00% |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX III (FCC III). |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 200 Aluminium : Less than 1 ppm. Lead : Less than 5 ppm. Arsenic : Less than 0.1 ppm. Mercury : Less than 0.1 ppm. Other heavy metals : Less than 20 ppm. |
| RESIDUAL SOLVENTS | Methylalcohol : Less than 20 ppm. |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g. Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100 Kg. inside a fiber drum. |
| AVAILABILITY | 1000 Kg/month. |

# SPECIFICATION SHEET

| PRODUCT | CARMINEX™ ACID 96% PROTEIN FREE |
| --- | --- |
| DESCRIPTION | It is the purest free colouring matter Carminic Acid with a concentration not less than 96%, obtained from a successive crystallisation of a cochineal aqueous-alcoholic extract (Dactylopius Coccus Costa), after eliminating proteins, resins, fats ans waxes. It is free of proteins and metals like calcium and aluminum. This is the difference with carmine. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ Acid 96% Protein Free is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SYNONYMS | Natural Red 4. C.I. 75470. Chemical Abstracts Service (CAS) registry number 1260-17-9. EEC CODE E 120. |
| USE | It is used as colourant in foods, drugs, cosmetics and also as a raw material for acid stable red colour production. Its protein free characteristics makes this product suitable for applications where allergenic reactions are not desired. |
| SOLUBILITY | Carminex™ Acid 96% Protein Free is 100% soluble in water in all range of pH, in alcohol and propylene glycol. |
| STABILITY | Highly resistant to light oxidation, change in pH, microbiological attacks. Very stable toward heat up to 130 centigrade degrees, but unstable above that temperature. |
| APPLICATION | Food Industry: jams, sauces, confectionery, alcoholic beverage, soft drinks. The FDA list the carmine among colours permitted in foods, without limitation and exempt from certification. |
| CONSERVATION | Two years at normal temperature. Store in a cold and dry location (5-15ºC). No refrigeration needed. Prevent from humidity. |
| COMPOSITION | Carminic Acid 96.00% Moisture 4.00% |
| ANALYSIS | All analysis approved under the FOOD CHEMICAL CODEX III (FCC III). |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 200 Protein content : Less than 0.2% Aluminium : Less than 1 ppm. Lead : Less than 5 ppm. Arsenic : Less than 0.1 ppm. Mercury : Less than 0.1 ppm. Other heavy metals : Less than 20 ppm. |
| RESIDUAL SOLVENTS | Methylalcohol : Less than 20 ppm. |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g. Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100 Kg. inside a fiber drum. |
| AVAILABILITY | 1000 Kg/month. |

## SPECIFICATION SHEET

| PRODUCT | CARMINEX™ RED ACID STABLE 40% |
| --- | --- |
| DESCRIPTION | It is an organic natural colour obtained from Cochineal extractions (Dactylopius coccus Costa). It is an Ammonium Carminate stable at acid pH which active agent is the Carminic Acid. This product is free of aluminium and calcium, and his main characteristic is to maintain a red stable colour in acidic media. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ Red Acid Stable 40% is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SOLUBILITY | It is a product easily soluble in water and in alcoholic solutions, keeping in both cases a red shade. |
| STABILITY | Good stability to the changes of pH, maintaining a red shade between pH 1 to 9. Besides, it shows good stability to light and heat. |
| pH (1%) | 6 – 7. |
| APPLICATION | Carminex™ Red Acid Stable 40% is a product specially developed to be used in beverages. Its main characteristic is the stability of its red shade at the changes of pH. It may be used in soft drinks, alcoholic drinks, and some other product where a red colour is needed. |
| CONSERVATION | Two years at normal temperature. Product should be stored in a dry and cold location (between 5 and 15°C). |
| CONCENTRATION | Not less than 40% Carminic Acid. Its concentration is adjusted with natural carriers GMO free. |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 200 Moisture : Less than 10% Aluminium : Less than 10 ppm. Lead : Less than 5 ppm. Arsenic : Less than 1 ppm. Mercury : Less than 1 ppm. Other heavy metals : Less than 20 ppm. |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g. Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100 Kg. inside a fibre drum. |
| AVAILABILITY | 1000 Kg/month. |

## SPECIFICATION SHEET

| PRODUCT | CARMINEX™ RED ACID STABLE 6.5% LIQUID |
| --- | --- |
| DESCRIPTION | It is an organic natural colour obtained from Cochineal extractions (Dactylopius coccus Costa). It is an Ammonium Carminate stable at acid pH which active agent is the Carminic Acid. This product is free of aluminium and calcium, and its main characteristic is to maintain a red stable colour in acidic media. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Carminex™ Red Acid Stable 6.5% Liquid is in full compliance with the directives of the European Union Colour n° E 120, the National Formulary and 21 CFR 73.100 for Cochineal derivatives. |
| SOLUBILITY | It is a product easily miscible in water and in alcoholic solutions, keeping in both cases a red shade. |
| STABILITY | Good stability to the changes of pH, maintaining a red shade between pH 1 to 9. Besides, it shows good stability to light and heat. |
| pH (1%) | 6 – 7. |
| APPLICATION | Carminex™ Red Acid Stable 6.5% Liquid is a product specially developed to be used in beverages. Its main characteristic is the stability of its red shade at the changes of pH. It may be used in soft drinks, alcoholic drinks, and some other product where a red colour is needed. |
| CONSERVATION | Two years at normal temperature. Product should be stored in a dry and cold location (between 5 and 15°C). |
| CONCENTRATION | Not less than 6.5% Carminic Acid. Its concentration is adjusted with natural carriers GMO free. |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 200 Moisture : Less than 10% Aluminium : Less than 10 ppm. Lead : Less than 5 ppm. Arsenic : Less than 1 ppm. Mercury : Less than 1 ppm. Other heavy metals : Less than 20 ppm. |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g. Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Sealed plastic bottles of 5, 20, 50 or 200 Kg. |
| AVAILABILITY | 5000 Kg/month. |

# SPECIFICATION SHEET

| PRODUCT | BIXINEX™ 90% |
| --- | --- |
| DESCRIPTION | It is the main natural pigment (Bixin) present in annatto seeds (Bixa orellana L.) solvent extracted in the form of a brown-violet powder. It can be available up to 80% of concentration. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Bixinex™ 90% fully complies with the requirements of the U.S. Code of Federal Regulations (Chapter 21, Part 1-99 Section 73.30) for Annatto Colours and is exempt from U.S. certification. |
| CONCENTRATION | Not less than 90% (FAO/FDA analysis method). |
| SOLUBILITY | Completely soluble in aqueous alkaline. Slightly soluble in vegetal oil. |
| APPLICATION | Fats, oils, margarines, mayonnaise, processed cheeses, snacks, dried foods, baked food, salad oils. |
| CONSERVATION | Protect from exposure to light, air, humidity and excessive heat. Stored in well sealed polyethylene packages. |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 200 Moisture : Less than 8% Mercury : Not detected Lead : Less than 5 ppm Arsenic : Less than 1 ppm Other heavy metals : Less than 20 ppm |
| RESIDUAL SOLVENTS | METHANOL : Less than 10 ppm N-HEXANE : Less than 10 ppm |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100kg inside a fibre drum. |
| AVAILABILITY | 1000/Kg. month. |

# SPECIFICATION SHEET

| PRODUCT | N BIXINEX™ 70% |
| --- | --- |
| ### DESCRIPTION | ## N Bixinex™ 70% is a dark orange Norbixin powder obtained from hydrolysed Bixin (main pigment of annatto seeds bixa orellana L.). |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | N Bixinex™ 70% fully complies with the requirements of the U.S. Code of Federal Regulations (Chapter 21, Part 1-99 Section 73.30) for Annato Colours and is exempt from U.S. certification. |
| CONCENTRATION | Not less than 70% (FAO / FDA analysis method). |
| SOLUBILITY | Completely soluble in aqueous alkali without any oil material residue or sediment. Insoluble in oil. |
| STABILITY | Moderately stable toward light and heat. It has good stability against oxidation, change in pH, and microbiological attacks. |
| APPLICATION | Food industry (alcoholic beverages, dairy desserts, dry pet foods, snack foods, processed cheese, ice cream, baked goods, icings, seafood). |
| CONSERVATION | Protect from exposure to light, air, humidity and excessive heat. Stored in well sealed polyethylene packages. |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 100 Moisture : Less than 8% Mercury : Not detected Lead : Less than 5 ppm Arsenic : Less than 1 ppm Other heavy metals : Less than 10 ppm |
| RESIDUAL SOLVENTS | METHANOL : Less than 10 ppm N-HEXANE : Less than 10 ppm |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100Kg. inside a fibre drum. |
| AVAILABILITY | 2000 Kg/month. |

# SPECIFICATION SHEET

| PRODUCT | N BIXINEX™WATERSOLUBLE 30% |
| --- | --- |
| DESCRIPTION | N Bixinex™ Water Soluble 30% is an orange Hydrosoluble Norbixin powder obtained from spray dried alkaline solution of hydrolysed Bixin. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | N Bixinex™ WaterSoluble 30% fully complies with the requirements of the U.S. Code of Federal Regulations (Chapter 21, Part 1-99 Section 73.30) for Annatto Colours and is exempt from U.S. certification. |
| CONCENTRATION | Not less than 30% (FAO/FDA analysis method). |
| SOLUBILITY | Completely soluble in water (crystalline) without any oil material residue or sediment. |
| APPLICATION | Food industry (alcoholic beverages, dairy desserts, dry pet foods, snack foods, processed cheese, ice cream, baked foods, icings, seafood). |
| CONSERVATION | Protect from exposure to light, air, humidity and excessive heat. Stored in well sealed polyethylene packages. |
| PHYSICO-CHEMICAL SPECIFICATIONS | Particle Size : 100% mesh 100 Moisture : Less than 8% Mercury : Not detected Lead : Less than 5 ppm Arsenic : Less than 1 ppm Other heavy metals : Less than 20 ppm |
| RESIDUAL SOLVENTS | METHANOL : Less than 10 ppm N-HEXANE : Less than 10 ppm |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Double polyethylene bags of 5, 25, 50 or 100 Kg. inside a fibre drum. |
| AVAILABILITY | 1000/Kg. month. |

# SPECIFICATION SHEET

| PRODUCT | BIXINEX™ 4% Oil dispersible |
| --- | --- |
| DESCRIPTION | It is the main natural pigment (Bixin) present in annatto seeds (Bixa orellana L.) extracted directly with vegetable oil, in the form of a fine and stable suspension. It can be available up to 4% of concentration. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | Bixinex™ 4% Oil dispersible fully complies with the requirements of the U.S. Code of Federal Regulations (Chapter 21, Part 1-99 Section 73.30) for Annatto Colours and is exempt from U.S. certification. |
| CONCENTRATION | Not less than 4% (FAO/FDA analysis method). |
| APPLICATION | Fats, oils, butters, margarines, mayonnaise, processed cheeses, snacks, dried foods, baked food, salad oils. |
| CONSERVATION | Protect from exposure to light, air, humidity and excessive heat. Stored in well sealed polyethylene packages. |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Sealed plastic bottles of 5, 20, 50 or 200 kg |
| AVAILABILITY | 5000/Kg. month. |

# SPECIFICATION SHEET

| PRODUCT | N BIXINEX™ LIQUID 1.2% |
| --- | --- |
| DESCRIPTION | N Bixinex™ Liquid 1.2% is a dark orange liquid solution of Norbixin in alkaline medium. |
| CERTIFICATION | Sephardi Kosher Organization |
| APPROVAL | N Bixinex™ Liquid 1.2% fully complies with the requirements of the U.S. Code of Federal Regulations (Chapter 21, Part 1-99 Section 73.30) for Annato Colours and is exempt from U.S. certification. |
| CONCENTRATION | Not less than 1.2% (FAO / FDA analysis method). |
| SOLUBILITY | Completely miscible in water without any oil material residue or sediment. Insoluble in oil. |
| STABILITY | Moderately stable toward light and heat. It has good stability against oxidation, change in pH, and microbiological attacks. |
| APPLICATION | Food industry (alcoholic beverages, dairy desserts, dry pet foods, snack foods, processed cheese, ice cream, baked goods, icings, seafood). |
| CONSERVATION | Protect from exposure to light, air, humidity and excessive heat. Store in well-sealed plastic bottles . |
| PHYSICO-CHEMICAL SPECIFICATIONS | Mercury : Not detected Lead : Less than 5 ppm Arsenic : Less than 1 ppm Other heavy metals : Less than 10 ppm pH : 11 – 11.50 |
| RESIDUAL SOLVENTS | METHANOL : Less than 10 ppm N-HEXANE : Less than 10 ppm |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Sealed plastic bottles of 5, 20, 50 or 200Kg. . |
| AVAILABILITY | 5000 Kg/month. |

##### SPECIFICATION SHEET

| PRODUCT | PAPRINEX™ OLEORESIN 100 000 |
| --- | --- |
| DESCRIPTION | ###### Natural liquid extract obtained by extraction with adequate organic solvent from the dried ripe fruit of Capsicum Annum L. and subsequent removal of solvent. Gives a deep dark red color in oil, viscous and homogeneous. |
| CERTIFICATION | Sephardi Kosher Organization |
| SYNONYMS | ### Chemical Abstract Service (CAS) registry number 68917 – 78 – 2. EEC CODE E – 160c |
| COLOR | Not less than 2450 ASTA (20.1 Method ) Not less than 100000 U.C. |
| SMELL AND TASTE | Typical of the raw material |
| DENSITY | 0.93 a 0.97g/ml at 20ºC |
| SOLUBILITY | Soluble in oil |
| APPLICATION | ### Food industry: meat products, sausages, snacks, sauces, processed cheese, spice blends, breading. |
| CONSERVATION | ### Extract should be held at temperatures lower than 15ºC and protected from light, heat and freezing.. |
| RESIDUAL SOLVENT | Hexane : Less than 25 ppm |
| PHYSICO-CHEMICAL SPECIFICACIONS | Humidity : Less than 0.5% Sediments : Less than 2% Arsenic : Less than 3 ppm Lead : Less than 5 ppm Mercury : Less than 1 ppm Cadmium : Less than 1 ppm |
| MICROBIOLOGICAL STANDARD | Total plate count : Less than 1000/g Mould count : Less than 100/g. Yeast count : Less than 100/g. Coliforms : Less than 3/g. Salmonella : Not detected Vibrio-cholera : Not detected E coli : Not detected |
| PACKING | Sealed plastic bottles of 5, 20, 50 or 200 Kg. |
| AVAILABILITY | 4000 Kg/Month |